OPTICAL FIBER CABLE DATA SHEET

# CABLE CODE/CABLE TYPE

A-DQ(BN)2Y MLT mxn

## MULTI LOOSE TUBE, NON METALIC ARMOURED , HDPE JACKET FIBER OPTIC CABLE

# KEY FEATURES

Non Metallic Armoured, Jelly Filled Loose Tube, UV- Resistant, Rodent Resistant, Waterproof, PE JACKET

# CABLE DRAWING and CROSS SECTION

STANDARTS

IEC 60794-1&2,

# CONSTRACTIONS

Fiber Type			G.652-D,G.657 A1,G.657 A2,OM1,OM2,OM3,OM4,OM5

Fiber Count	        	12-144

Jacket Color			According to customer request

Jacket UV Resistance			Carbon Black

CPR Class					Fca

# APPLICATIONS

| Inter Building Voice or Data Communication Networks |  |
| --- | --- |
| FTTC Cabling |  |
| Outside Plant |  |
| Installed in Ducts & Conduits |  |
| Primary & Secondary Distribution |  |
| WAN |  |
| FTTB Cabling, |  |
| Campus Cabling |  |
| Directly Buried Underground |  |
| Telecommunication Data Trunk Systems |  |
| Harsh Premises Environments requiring heavy duty protection |  |
| Interconnection of Distribution Boxes, Distribution Frames and the Panels at Customer Side |  |

# DIMENSIONS/PHYSICAL SPECIFICATIONS

|  |  |  |  |  |  |  |  |  | IEC 60794-1-2 Test method |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of fibers | 12 | 24 | 36 | 48 | 60 | 72 | 96 | 144 |  |
| Number of tubes | 1 | 2 | 3 | 4 | 5 | 6 | 8 | 12 |  |
| Number of dummy Tubes | 5 | 4 | 3 | 2 | 1 | 0 | 0 | 0 |  |
| OD Tube Max. (mm) | 2,3 |  |  |  |  |  |  |  |  |
| OD Tube Nom. (mm) | 2,1 |  |  |  |  |  |  |  |  |
| OD Tube Min. (mm) | 1,9 |  |  |  |  |  |  |  |  |
| OD cable Max. (mm) | 11,0 |  |  |  |  |  | 13,5 | 16,5 |  |
| OD cable Nom. (mm) | 10,5 |  |  |  |  |  | 13,0 | 16,0 |  |
| OD cable Min. (mm) | 10,0 |  |  |  |  |  | 12,5 | 15,5 |  |
| Max. Jacket Thickness (mm) | 1,8 |  |  |  |  |  |  |  |  |
| Min. Jacket Thickness (mm) | 1,5 |  |  |  |  |  |  |  |  |
| Minimum Bending radius | 20 x D |  |  |  |  |  |  |  | E11 |
| Max. Operating Load (N) | 1600 |  |  |  |  |  |  |  |  |
| Maximum Installation Pulling Load (N) | 2700 |  |  |  |  |  |  |  |  |
| Crush (N/100mm) | 2000 |  |  |  |  |  |  |  | E3 |
| Twist (torsion) | 5 turn of 180° on 1m sample, both ways. |  |  |  |  |  |  |  | E7 |
| Impact resistance |  |  |  |  |  |  |  |  | E4 |
| Up to 15mm OD | 4.4J ; 3 x 2 times |  |  |  |  |  |  |  |  |
| 15 to 17.5mm OD | 7J ; 3 x 2 times |  |  |  |  |  |  |  |  |
| 17 to 20mm OD | 10J ; 3 x 2 times |  |  |  |  |  |  |  |  |
| Over 20mm OD | 13J ; 3 x 2 times |  |  |  |  |  |  |  |  |
| Storage temperature Range | -40°C to +70°C |  |  |  |  |  |  |  | F1 |
| Operating temperature Range | -40°C to +70°C |  |  |  |  |  |  |  | F1 |
| Core fluid Penetration | 3m sample, 1m head for 24 hours |  |  |  |  |  |  |  | F5 |
| Nom. Weight (Kg/Km) | 90 |  |  |  |  |  | 120 | 180 |  |

QUALIFICATION SPECIFICATIONS

Cable Qualification Standards	ANSI/ICEA S-104-696   |   EN 187105   |   Telcordia GR-20   |   Telcordia  GR-409

# CERTIFICATIONS

RoHS	Compliant

ISO 9001:2008	Designed, manufactured and/or distributed under this quality management  system

### ISO 14001:2004            Designed, Environmental management systems

ISO 18001:2007 	Occupational Health and Safety Assessment Series

PACKING and DELIVERY LENGTHS

WOODEN DRUMS, standart delivery lengths are 4000mt

SHEATH MARKING

INFOKS KABLO  WEEK/YEAR  A-DQ(BN)2Y xxxF   TYPE of FIBRE    DRUM NO  LENGTH (M)